CS 350 Final Thermostat Report

Smart Thermostat Prototype and Architecture

Joseph Toj

Southern New Hampshire University

CS-350-10526-M01 Emerging Sys Arch & Tech

Instructor: Sina Asadi

February 2026

SysTec Embedded Prototype Report

SysTec has approached us about developing a product which is a residential smart thermostat with the ability to adjust temperature and eventually communicate with an external cloud server. The goal of this project was to build an embedded prototype that exemplifies thermostat temperature control logic, peripheral integration, user interaction, and simulated cloud connectivity.

The prototype was built on a Raspberry Pi 4B using an AHT20 temperature sensor communicating over I2C. The device will read ambient room temperature at regular intervals and use the input to control user-configurable heating and cooling set points (default is 72F). 3 GPIO pushbuttons are provided to allow user control of system behavior. One button is used to cycle through the operating modes, while the other two buttons increase or decrease the temperature set point by one degree.

Thermostat behavior is handled as finite state machine with 3 modes: OFF, HEAT, COOL.

OFF State: Output for heat and cool are disabled. Both red and blue LEDs are off.

HEAT: If the current temperature is less than the set point, the red LED pulses slowly to indicate heating is on. If the current temperature is greater than or equal to the set point, the red LED is solid to indicate heating is off.

COOL: If the current temperature is greater than the set point, the blue LED pulses slowly to indicate cooling is on. If the current temperature is less than or equal to the set point, the blue LED is solid to indicate cooling is off.

User feedback is displayed on the LCD. The top line always displays the current date and time. The bottom-line cycles between displaying the current temperature and displaying which operating mode is active with the set point.

To demonstrate communication with SysTec’s analytics platform, a comma delimited string is transmitted over UART every 30 seconds in the following format:

state,current_temperature,set_point

Future development will connect to a cloud server and send device telemetry for data analysis and monitoring.

Hardware Architecture Analysis

Although a Raspberry Pi was used for this prototype, there are many alternatives for embedded hardware architectures to consider when bringing this product to production.

The Raspberry Pi meets all peripheral requirements (I2C, UART, GPIO, PWM) and even has built-in Wi-Fi available on many models. Additionally, Raspberry Pis provide ample RAM and onboard storage. The biggest downside of the Pi is that it is a Linux based SBC with cost and power requirements that exceed the needs of a thermostat device.

Microchip offers a wide range of microcontrollers designed for embedded and IoT products. Microchip’s microcontrollers support all the needed interfaces (I2C, UART, GPIO, PWM) and have options with built-in Wi-Fi or companion Wi-Fi modules. Microcontrollers are extremely power efficient and offer enough Flash and RAM to store thermostat firmware and support network connectivity.

Freescale (now NXP) offers another great option for microcontrollers. These are ARM-based microcontrollers specifically optimized for real-time embedded control systems. Like Microchip, these devices also include robust peripheral support and configurable memory capacity. Many NXP-based platforms offer IoT-ready Wi-Fi capabilities.

Recommendation

A Raspberry Pi works great for prototyping, but Microchip or NXP microcontrollers with Wi-Fi support are recommended for production. Both provide peripheral compatibility, Wi-Fi capabilities, and enough Flash and RAM to support control logic and allow for cloud communications. Since the core of the control software was designed around a finite state machine, it can easily be adapted to run on a microcontroller with minor change to the logic. This will allow SysTec to easily develop a production-ready smart thermostat.